# ABSTRACT

Design the automated sorting machine using conveyor belt to need the manufacturing industry in many fields is a very complex process. The system needs to develop in order to meet the industry satisfactions. This is an industrial automation based application. It shows the concept of normal conveyor belt, but with some intelligence. We can also call it as intelligent conveyor belt, as it has also ability to sort the object of different sizes. This helps to avoid size malfunctioning in production machines. In the core of the project, Embedded System (NodeMCU) and Internet of Things technology have been used. The NodeMCU is brain of the system and controls the relay and drives servo motor according to input from size sorting sensors. IR rays in photodiode are used for detection. The objects of different sizes are passed through the sensors and the object having specified size is sorted. The belt is driven by drive circuit which is controlled by Push button. The internet of things technology provides the monitoring of the process and person can observe the count on smart phone. By developing such sorting system the production rate of the manufacturing industry has been increased since these sorting systems replaced the human resources.

Contents

ABSTRACT	1

CHAPTER 1	5

1.	Introduction	5

CHAPTER 2	7

2.	Internet of Things Enabled Size Sorting Machine	7

2.1	Objective	7

2.2	Methodology	7

2.3	Components required	9

2.3.1	Conveyor	9

2.3.2	IR SENSOR	10

2.3.2.1	Types of IR Sensors	11

2.3.2.2	IR Transmitter	12

2.3.2.3	IR Receiver	13

2.3.3	Servo Motor	15

2.3.4	DC Motor	18

2.3.5	L293D Motor Driver Module	22

CHAPTER 3	25

3	Customer Cloud Architecture for Internet of Things	25

3.1	Fundamental to IoT	26

3.2	Aspects of the architecture include:	29

3.3	Cloud Components	31

3.4	User Layer	32

3.5	Proximity Network	32

3.6	User Interface	33

3.7	IoT Gateway	34

3.8	Device Data Store	34

3.9	Provider Cloud	34

3.10	IoT Transformation and Connectivity	35

3.11	Application Logic	36

3.12	Visualization	36

CHAPTER 4	46

4.	NODEMCU and Its Configuration with Arduino IDE	46

4.1	NodeMCU	46

4.2	NodeMCU Specification	49

4.3	Install the Arduino IDE 1.6.4 or greater	49

4.4	Install the ESP8266 Board Package	50

4.5	NodeMCU Configuration with Arduino IDE	50

CHAPTER 5	60

5.	Introduction and Interfacing of Blynk App with Device	60

5.1	Introduction to Blynk	60

5.2	Features	61

5.3	Blynk Configuring	61

5.4	Getting Started with The Blynk App	62

5.4.1	Create a Blynk Account	62

5.4.2	Create a New Project	63

5.4.3	Authentication Token	64

5.4.4	Add a Widget	67

5.5	Run Project	70

5.6	Virtual Pin Operations	70

5.7	Sending Data from APP to Hardware	71

5.8	Pushing Data from Hardware	72

5.9	Controlling of Multiple Devices	72

5.10	Widgets	73

5.10.1	Common Widgets Setting	74

5.11	Controllers	74

CONCLUSION	77

REFERENCES	78

Appendix-A	79

Source Code	79

List of Figures

Figure 1 Circuit Diagram of implementation	8

Figure 2 Conveyor	9

Figure 3 IR Sensor working	12

Figure 4 IR Transmitter	12

Figure 5 Schematic of IR Transmitter	13

Figure 6 IR Receiver	14

Figure 7 IR Receiver circuit	14

Figure 8 Gear DC Motor	17

Figure 9 Gear DC Motor	19

Figure 10 Fleming’s Left Hand Rule	20

Figure 11 Principle of operation of DC Motor	21

Figure 12 H-Bridge	23

Figure 13 LM293D Pin details	24

Figure 14 Elements of IoT Solutions	29

Figure 15  Cloud Components for IoT	31

Figure 16  Complete Picture of IoT Cloud	42

Figure 17  NodeMCU PINs Details	48

Figure 18  Adding NodeMCU URL	51

Figure 19  NodeMCU Packages Installation	51

Figure 20  ESP8266 Packages Installation	52

Figure 21  Selection of NodeMCU	53

Figure 22  Selection of Frequency	54

Figure 23 Selection of Baud Rate	55

Figure 24  COM Poet Seeting in Device Manager	56

Figure 25  COM Port Selection in Arduino Ide	57

Figure 26  Verification of Sketch	58

Figure 27  Uploading of Sketch into NodeMCU	59

Figure 28 Blynk Cloud Architecture	61

Figure 29  New Account Creation	63

Figure 30  New Project Creation	64

Figure 31  New Project Name and Device Selection	64

Figure 32  New Project Setting	65

Figure 33   Addition of More Device in same project	66

Figure 34  Authentication Token	66

Figure 35  Project Creation	67

Figure 36  Blynk Widgets	68

Figure 37  Blynk Widgets Button Selection	69

Figure 38  Blynk Widgets Button Settings	69

Figure 39  Blynk Widgets Button online	70

Figure 40  Blynk Widgets Button Selection	72

Figure 41  Blynk Widget Pushing Data	72

Figure 42  Blynk Widgets Button Selection	73

# CHAPTER 1

## Introduction

The fruit and vegetable market is getting highly selective, requiring their suppliers to distribute the goods according to high standards of quality and presentation. In the last years, a number of fruit sorting and grading systems have appeared to fulfill the needs of the fruit processing industry. Present sorting systems tend to include the development of an electronic weight system and a vision-based sorting and grading unit which also measures size, with a friendly user interface that enables definition of classification parameters, reconfiguration of the outputs and maintenance of production statistics. Some commercially available systems are approaching this objective, but prices are becoming almost prohibitive for small and medium companies that try to maintain competitive levels. This is the case of many world fruit packing companies, which are usually small, agriculture products are quite price-sensitive, and they suffer from a hard competitive market. Thus, the idea was to build a new system integrating in a flexible way all parts (mechanics, control, weight and vision) of a fruit sorter. From the very beginning there was the criterion that the system should be conceived as an open platform ready to evolve and incorporate, without major changes, new requirements from the customers or simply an upgrade of any of its modules to avoid the obsolescence of its design or components.

Such Sorting system is extensively used in many industries like food processing industries, toy industries, etc to ensure that the quality of the product is up to the mark. This process is simplified by the use of automation. Automation is the use of control systems like computers or robots for handling different process and machineries to replace a human being and provides mechanical assistance. Automated systems generally use more complex algorithms which increase the cost of the design and the power consumed. This not only reduces manual efforts, time consumed, gives more time for marketing, but also prevents danger which might occur when human beings work in hazardous environments. Automation greatly improves the productivity and is highly scalable. The purpose of this project is to save the time for inspection and to reduce the efforts of the workers in material handling.

An automatic sorting machine has main task of sorting components according to the sizes. This also consist of conveyor belt, which reduces the efforts of material handling. Also both processes take place simultaneously viz material handling and inspection.

A sorting machine is more practical and economical method of automation, which transfers material from one point to another. The design is quite simple and of flexible use, means only conveyor belt can be used for material handling. Apart from the electronic system, mechanical system, Internet of things technology has also been added in which is adding new dimensions to sorting industry, Monitoring of Process, analysis of present things and based on that prediction can be done for future requirement and the can control the working process.

# CHAPTER 2

## Internet of Things Enabled Size Sorting Machine

## Objective

Manual sorting is the conventional approach that is preferred by industries which involves visual observation performed by human beings. This is an approach where human laborers are made to work for maximum time to achieve the desired task. When we consider the large scale industries, segregation of objects that are bulk in number becomes a tedious task for laborers which consume lot of time. Recognizing a particular object and placing it in a required place is a tiring work, wherein one has to sort a bulk of objects with greater weight in quick time. This is slow and non-consistent when the human laborers do it manually. Sometimes humans are restricted to work in the hazardous conditions and it is the place where automation plays a major role.

## Methodology

- The objects are fed to the conveyor at stipulated time intervals.

- This in turn initializes the DC motor which will move the object along the conveyor line.

- At the starting of conveyor belt we have fixed a stand and fixed two IR sensors to it one upon another. The logic behind this is if both lower and upper IRs are high then the object is determined as big and if only the lower IR is high then the object is determined as small.

- At the end of the conveyor we have fixed a servo motor to divert the way of objects to their respective bins. we have placed two bins at the edge of conveyor at angles of 30 and 140 degrees respectively. if the object is small servo motor rotates 30o and if the object is big then the servo motor rotates 140o . when servo motor rotates 30o the long shaft attached to it diverts the way of object to bin of containing small objects. when servo motor rotates 140o the long shaft attached to it diverts the way of object to bin of containing big objects.

- Now the part of counting objects is based on the program written by us. We initialize two variables small and big as zero initially. Whenever both IRs detect the object then big variable increments and whenever only lower IR detects the object then small variable increments.

Figure 1 Circuit Diagram of implementation

- The NodeMCU microcontroller using in this project has inbuilt Wi-Fi shield named ESP8266. Using this Wi-Fi shield microcontroller will connect to Wi-Fi through username and password specified in program. Through Wi-Fi microcontroller will connect to Blynk cloud using the authentication token specified in program. After this we can see the device online in our authentication token related project in Blynk app. For this project we are using two value display widgets in Blynk one for small counter and one for big counter. we are sending the counter values of big and small variables to two widgets through virtual pins V2 and V3 respectively.

- By using the above process, we can monitor our production quantity from anywhere through internet.

- Internet of things technology helps for centralized monitoring of the entire process. Based on the statistics collected from process can be used for analysis and doing the prediction for business.

## Components required

- Conveyor

- NodeMCU(Discussed in chapter 4)

- Motors (x2)

- IR sensor (x2)

- Servo motor

## Conveyor

A conveyor belt consists of two or more pulleys, with a continuous loop of material which rotates over them. One or both of the pulleys are powered, moving the belt and material on the belt forward. There are two main industrial classes of belt conveyors; those in general material handling such as those moving boxes along inside a factory and bulk material handling such as those used to transport industrial and agricultural materials, such as grain, coal, ores etc.

Figure 2 Conveyor

Rubber conveyor belts are commonly used to convey items with irregular bottom surfaces, small items that would fall in between rollers (e.g. a sushi conveyor bar), or bags of product that would sag between rollers. Belt conveyors are generally fairly similar in construction consisting of a metal frame with rollers at either end of a flat metal bed. The belt is looped around each of the rollers and when one of the rollers is powered (by an electrical motor) the belting slides across the solid metal frame bed, moving the product. In heavy use applications the beds which the belting is pulled over are replaced with rollers. The rollers allow weight to be conveyed as they reduce the amount of friction generated from the heavier loading on the belting.

## IR SENSOR

Infrared technology addresses a wide variety of wireless applications. The main areas are sensing and remote controls. In the electromagnetic spectrum, the infrared portion is divided into three regions: near infrared region, mid infrared region and far infrared region.

The wavelengths of these regions and their applications are shown below.

- Near infrared region — 700 nm to 1400 nm — IR sensors, fiber optic

- Mid infrared region — 1400 nm to 3000 nm — Heat sensing

- Far infrared region — 3000 nm to 1 mm — Thermal imaging

The frequency range of infrared is higher than microwave and lesser than visible light. For optical sensing and optical communication, photo optics technologies are used in the near infrared region as the light is less complex than RF when implemented as a source of signal. Optical wireless communication is done with IR data transmission for short range applications.

An infrared sensor emits and/or detects infrared radiation to sense its surroundings.

The working of any Infrared sensor is governed by three laws:

- Planck’s Radiation law

- Stephen – Boltzmann law

- Wien’s Displacement law.

Planck’s law states that “every object emits radiation at a temperature not equal to 00K”. Stephen – Boltzmann law states that “at all wavelengths, the total energy emitted by a black body is proportional to the fourth power of the absolute temperature”. According to Wien’s Displacement law, “the radiation curve of a black body for different temperatures will reach its peak at a wavelength inversely proportional to the temperature”.

The basic concept of an Infrared Sensor which is used as Obstacle detector is to transmit an infrared signal, this infrared signal bounces from the surface of an object and the signal is received at the infrared receiver.

There are five basic elements used in a typical infrared detection system: an infrared source, a transmission medium, optical component, infrared detectors or receivers and signal processing. Infrared lasers and Infrared LED’s of specific wavelength can be used as infrared sources. The three main types of media used for infrared transmission are vacuum, atmosphere and optical fibers. Optical components are used to focus the infrared radiation or to limit the spectral response.

Optical lenses made of Quartz, Germanium and Silicon are used to focus the infrared radiation. Infrared receivers can be photodiodes, phototransistors etc. some important specifications of infrared receivers are photosensitivity, directivity and noise equivalent power. Signal processing is done by amplifiers as the output of infrared detector is very small.

## Types of IR Sensors

Infrared sensors can be passive or active. Passive infrared sensors are basically Infrared detectors. Passive infrared sensors do not use any infrared source and detects energy emitted by obstacles in the field of view. They are of two types: quantum and thermal. Thermal infrared sensors use infrared energy as the source of heat and are independent of wavelength. Thermocouples, pyroelectric detectors and bolometers are the common types of thermal infrared detectors.

Quantum type infrared detectors offer higher detection performance and are faster than thermal type infrared detectors. The photosensitivity of quantum type detectors is wavelength dependent. Quantum type detectors are further classified into two types: intrinsic and extrinsic types. Intrinsic type quantum detectors are photoconductive cells and photovoltaic cells.

Active infrared sensors consist of two elements: infrared source and infrared detector. Infrared sources include an LED or infrared laser diode. Infrared detectors include photodiodes or phototransistors. The energy emitted by the infrared source is reflected by an object and falls on the infrared detector.

Figure 3 IR Sensor working

## IR Transmitter

Infrared Transmitter is a light emitting diode (LED) which emits infrared radiations. Hence, they are called IR LED’s. Even though an IR LED looks like a normal LED, the radiation emitted by it is invisible to the human eye. The picture of a typical Infrared LED is shown below.

Figure 4 IR Transmitter

There are different types of infrared transmitters depending on their wavelengths, output power and response time.

A simple infrared transmitter can be constructed using an infrared LED, a current limiting resistor and a power supply. The schematic of a typical IR transmitter is shown below.

Figure 5 Schematic of IR Transmitter

When operated at a supply of 5V, the IR transmitter consumes about 3 to 5 mA of current. Infrared transmitters can be modulated to produce a particular frequency of infrared light. The most commonly used modulation is OOK (ON – OFF – KEYING) modulation.

IR transmitters can be found in several applications. Some applications require infrared heat and the best infrared source is infrared transmitter. When infrared emitters are used with Quartz, solar cells can be made.

## IR Receiver

Infrared receivers are also called as infrared sensors as they detect the radiation from an IR transmitter. IR receivers come in the form of photodiodes and phototransistors. Infrared Photodiodes are different from normal photo diodes as they detect only infrared radiation. The picture of a typical IR receiver or a photodiode is shown below.

Figure 6 IR Receiver

Different types of IR receivers exist based on the wavelength, voltage, package, etc. When used in an infrared transmitter – receiver combination, the wavelength of the receiver should match with that of the transmitter.

A typical infrared receiver circuit using a phototransistor is shown below.

Figure 7 IR Receiver circuit

It consists of an IR phototransistor, a diode, a MOSFET, a potentiometer and an LED. When the phototransistor receives any infrared radiation, current flows through it and MOSFET turns on. This in turn lights up the LED which acts as a load. The potentiometer is used to control the sensitivity of the phototransistor.

Distinguishing Between Black and White Colors

It is universal that black color absorbs the entire radiation incident on it and white color reflects the entire radiation incident on it. Based on this principle, the second positioning of the sensor couple can be made. The IR LED and the photodiode are placed side by side. When the IR transmitter emits infrared radiation, since there is no direct line of contact between the transmitter and receiver, the emitted radiation must reflect back to the photodiode after hitting any object. The surface of the object can be divided into two types: reflective surface and non-reflective surface. If the surface of the object is reflective in nature i.e. it is white or other light color, most of the radiation incident on it will get reflected back and reaches the photodiode. Depending on the intensity of the radiation reflected back, current flows in the photodiode.

If the surface of the object is non-reflective in nature i.e. it is black or other dark color, it absorbs almost all the radiation incident on it. As there is no reflected radiation, there is no radiation incident on the photodiode and the resistance of the photodiode remains higher allowing no current to flow. This situation is similar to there being no object at all. The positioning and enclosing of the IR transmitter and Receiver is very important. Both the transmitter and the receiver must be placed at a certain angle, so that the detection of an object happens properly. This angle is the directivity of the sensor which is +/- 45 degrees.

In order to avoid reflections from surrounding objects other than the object, both the IR transmitter and the IR receiver must be enclosed properly. Generally the enclosure is made of plastic and is painted with black color.

## Servo Motor

A servo motor is an electrical device which can push or rotate an object with great precision. If you want to rotate and object at some specific angles or distance, then you use servo motor. It is just made up of simple motor which run through servo mechanism. If motor is used is DC powered then it is called DC servo motor, and if it is AC powered motor then it is called AC servo motor. We can get a very high torque servo motor in a small and light weight packages. Doe to these features they are being used in many applications like toy car, RC helicopters and planes, Robotics, Machine etc.

Servo motors are rated in kg/cm (kilogram per centimeter) most hobby servo motors are rated at 3kg/cm or 6kg/cm or 12kg/cm. This kg/cm tells you how much weight your servo motor can lift at a particular distance. For example: A 6kg/cm Servo motor should be able to lift 6kg if the load is suspended 1cm away from the motors shaft, the greater the distance the lesser the weight carrying capacity. The position of a servo motor is decided by electrical pulse and its circuitry is placed beside the motor.

It consists of three parts:

- Controlled device

- Output sensor

- Feedback system

It is a closed loop system where it uses positive feedback system to control motion and final position of the shaft. Here the device is controlled by a feedback signal generated by comparing output signal and reference input signal.

Here reference input signal is compared to reference output signal and the third signal is produces by feedback system. And this third signal acts as input signal to control device. This signal is present as long as feedback signal is generated or there is difference between reference input signal and reference output signal. So the main task of servomechanism is to maintain output of a system at desired value at presence of noises.

Working principle of Servo Motors

A servo consists of a DC Motor a potentiometer, gear assembly and a controlling circuit. First of all we use gear assembly to reduce RPM and to increase torque of motor. Say at initial position of servo motor shaft, the position of the potentiometer knob is such that there is no electrical signal generated at the output port of the potentiometer. Now an electrical signal is given to another input terminal of the error detector amplifier. Now difference between these two signals, one comes from potentiometer and another comes from other source, will be processed in feedback mechanism and output will be provided in term of error signal. This error signal acts as the input for motor and motor starts rotating. Now motor shaft is connected with potentiometer and as motor rotates so the potentiometer and it will generate a signal. So as the potentiometer’s angular position changes, its output feedback signal changes. After sometime the position of potentiometer reaches at a position that the output of potentiometer is same as external signal provided. At this condition, there will be no output signal from the amplifier to the motor input as there is no difference between external applied signal and the signal generated at potentiometer, and in this situation motor stops rotating.

Controlling Servo Motor:

All motors have three wires coming out of them. Out of which two will be used for Supply (positive and negative) and one will be used for the signal that is to be sent from the MCU.

Servo motor is controlled by PWM (Pulse with Modulation) which is provided by the control wires. There is a minimum pulse, a maximum pulse and a repetition rate. Servo motor can turn 90 degree from either direction from its neutral position. The servo motor expects to see a pulse every 20 milliseconds (ms) and the length of the pulse will determine how far the motor turns. For example, a 1.5ms pulse will make the motor turn to the 90° position, such as if pulse is shorter than 1.5ms shaft moves to 0° and if it is longer than 1.5ms than it will turn the servo to 180°.

Figure 8 Gear DC Motor

Servo motor works on PWM (Pulse width modulation) principle means its angle of rotation is controlled by the duration of applied pulse to its Control PIN. Basically servo motor is made up of DC motor which is controlled by a variable resistor (potentiometer) and some gears. High speed force of DC motor is converted into torque by Gears. We know that WORK= FORCE X DISTANCE, in DC motor Force is less and distance (speed) is high and in Servo, force is High and distance is less. Potentiometer is connected to the output shaft of the Servo, to calculate the angle and stop the DC motor on required angle.

Servo motor can be rotated from 0 to 180 degree, but it can go up to 210 degree, depending on the manufacturing. This degree of rotation can be controlled by applying the Electrical Pulse of proper width, to its Control pin. Servo checks the pulse in every 20 milliseconds. Pulse of 1 ms (1 millisecond) width can rotate servo to 0 degree, 1.5ms can rotate to 90 degree (neutral position) and 2 ms pulse can rotate it to 180 degree.

All servo motors work directly with your +5V supply rails but we have to be careful on the amount of current the motor would consume, if you are planning to use more than two servo motors a proper servo shield should be designed.

## DC Motor

Almost every mechanical movement that we see today is accomplished by an electric motor. An electric motor takes electrical energy and produces mechanical energy. Electric motors come in various ratings and sizes. Some applications of large electric motors include elevators, rolling mills and electric trains. Some applications of small electric motors are robots, automobiles and power tools. Electric motors are categorized into two types: DC (Direct Current) motors and AC (Alternating Current) motors. The function of both AC and DC motors is same i.e. to convert electrical energy to mechanical energy.

The basic difference between these two is the power supply which is an AC source for AC motors and a DC source like a battery for DC motors. Both AC and DC electric motors consist of a stator which is a stationary part and a rotor which is a rotating part or armature of the motor. The principle of working of an electric motor is based on the interaction of magnetic field produced by the stator and the electric current flowing in the rotor in order to produce rotational speed and torque.

Figure 9 Gear DC Motor

There are different kinds of DC motors and they all work on the same principle. A DC motor is an electromechanical actuator used for producing continuous movement with controllable speed of rotation. DC motors are ideal for use in applications where speed control and servo type control or positioning is required. A simple BO DC motor that is used in this project is shown figure 2.3. As mentioned earlier, any motor consists of two parts viz. stator and rotor.

Working Principle of DC Motor

An electromechanical energy conversion device will take electrical energy at the input and produces a mechanical energy at the output side. There are three electrical machines that are extensively used for this task: a DC motor, an induction or asynchronous motor and a synchronous motor. Induction motor and synchronous motors are AC motors. In all the motors, the electrical energy is converted into mechanical when the magnetic flux linking a coil is changed. An electric motor takes electrical energy as input and converts into mechanical energy.

When the electrical energy is applied to a conductor which is placed perpendicular to the direction of the magnetic field, the result of the interaction between the electric current flowing through the conductor and the magnetic field is a force. This force pushes the conductor in the direction perpendicular to both current and the magnetic field, hence, the force is mechanical in nature. The value of the force can be calculated if the density of the magnetic field B, length of the conductor L and the current flowing in the conductor I, is known.

The force exerted on the conductor is given by

F = B×I×L Newtons

The direction of the motion of the conductor can be determined with the help of Fleming’s Left Hand Rule. Fleming’s Left Hand Rule is applicable to all electric motors. The figure representing Fleming’s Left-Hand Rule is shown in figure 10. When a conductor which is carrying current is placed in a magnetic field, a force acts on the conductor that is perpendicular to both the directions of magnetic field and the current.

Figure 10 Fleming’s Left Hand Rule

According to Fleming’s Left Hand Rule, the left hand thumb represents the direction of the force, the index finger represents the direction of the magnetic field and the middle finger represents the direction of the current.

A DC motor consists of two sets of coils called armature winding and field winding. Field winding is used to produce the magnetic field. A set of permanent magnets can also be used for this purpose. If field windings are used, it is an electromagnet. The field winding is the fixed part of the motor or a stator. The armature winding is rotor part of the motor. The rotor is placed inside of stator. The rotor or the armature is connected to the external circuit through a mechanical commutator.

Generally, Ferro magnetic materials are used to make both stator and rotor which are separated by air gap. The coil windings inside the stator are made of series or parallel connections of number of coils. The Copper windings are generally employed for both armature and field windings.

The principle of operation of a DC motor is explained below. Consider a coil placed in a magnetic field with a flux density of B Tesla. When the coil is supplied with direct current by connecting it to a DC supply, a current I flows through the length of the coil.

The electric current in the coil interacts with the magnetic field and the result is exertion of a force on the coil according to the Lorenz force equation. The force is proportional to the strength of the magnetic field and the current in the conductor.

Figure 11 Principle of operation of DC Motor

The same principle is used in DC motor and it consists of several coils that are wound on the armature and all the coils experience the same force. The result of this force is the rotation of the armature. The rotation of the conductor in the magnetic field will result in torque. The magnetic flux linking with the conductor is different at different positions of the coil in the magnetic field and these causes to induce an emf in the coil according to the Faradays laws of electromagnetic induction. This emf is referred to as back emf. The direction of this emf is opposite to the supply voltage which is responsible for current to flow in the conductor. Hence the total amount of current flowing in the armature is proportional to the difference between the supply voltage and the back emf.

A DC motor electrical equivalent circuit is divided into two circuits: field circuit and armature circuit. Field circuit is responsible for the magnetic field and is supplied with a separate DC voltage Vf. The resistance and inductance of the field winding are represented by Rf and Lf. As a result of the voltage, a current If is produced in the winding and it establishes the necessary magnetic field.

In the armature, a voltage VT is applied across the terminals of the motor and a current Ia flows in the armature circuit. The resistance of the armature winding is Ra and the total voltage induced in the armature is Eb.

Applying Kirchhoff’s Voltage Law in both the circuits,

Vf = If * Rf

VT = Ia * Ra + Eb

The torque developed in the motor is

T = K * Ia * Φ

Where k is constant depending on coil geometry and Φ is magnetic flux.

The electrical power of the motor is Eb * Ia.

The developed power which is the power converted to mechanical form is given below:

P = T * ωn where ω is the angular speed.

This is the total power that is delivered to the induced armature voltage and

Eb * Ia = T * ωn

## L293D Motor Driver Module

As the name suggests, L293D is a quadruple H-bridge, high current motor driver IC. It can be used to drive two motors at a time in both the directions with an output current of 600mA for each motor. L293D IC is designed to drive relays, DC motors, stepper motors and other inductive loads with high current and high voltage requirements.

The L293D IC receives signals from the microprocessor and transmits the relative signal to the motors. It has two voltage pins, one of which is used to draw current for the working of the L293D and the other is used to apply voltage to the motors.

The L293D switches it output signal according tothe input received from the microprocessor.
For Example: If the microprocessor sends a 1(digital high) to the Input Pin of L293D, then the L293D transmits a 1(digital high) to the motor from its Output Pin. An important thing to note is that the L293D simply transmits the signal it receives. It does not change the signal in any case.

The L293D is a 16 pin IC, with eight pins, on each side, dedicated to the controlling of a motor. There are 2 INPUT pins, 2 OUTPUT pins and 1 ENABLE pin for each motor. L293D consist of two H-bridge. H-bridge is the simplest circuit for controlling a low current rated motor.
The Theory for working of a H-bridge is given below.

Figure 12 H-Bridge

In the given diagram, the arrow on the left points to the higher potential side of the input voltage of the circuit. Now if the switches S1 & S4 are kept in a closed position while the switches S2 & S3 are kept in a open position meaning that the circuit gets shorted across the switches S1 & S4.  This creates a path for the current to flow, starting from the V input to switch S1 to the motor, then to switch S4 and then the exiting from the circuit. This flow of the current would make the motor turn in one direction.

The direction of motion of the motor can be clockwise or anti-clockwise, this is because the rotation of the motor depends upon the connection of the terminals of the motor with the switches. For simplicity, lets assume that in this condition the motor rotates in a clockwise direction.

Now, when S3 and S2 are closed then and S1 and S4 are kept open then the current flows from the other direction and the motor will now definitely rotates in counter-clockwise direction.

When S1 and S3 are closed and S2 and S4 are open then the ‘STALL’ condition will occur (The motor will break).

Figure 13 LM293D Pin details

Working Mechanism

Now depending upon the values of the Input and Enable the motors will rotate in either clockwise or anticlockwise direction with full speed (when Enable is HIGH) or with less speed (when Enable is provided with PWM). Let us assume for Left Motor when Enable is HIGH and Input 1 and Input 2 are HIGH and LOW respectively then the motor will move in clockwise direction.

# CHAPTER 3

## Customer Cloud Architecture for Internet of Things

The Internet of Things (IoT) is one of the most exciting and most dynamic areas of IT at the present time. IoT involves the linking of physical entities (“things”) with IT systems that derive information about or from those things which can be used to drive a wide variety of applications and services which may be directly or indirectly connected or related to those things. IoT covers a very wide spectrum of applications, spanning enterprises, governments and consumers and represents the integration of systems from traditionally different communities: Information Technology and Operational Technology. As a result, it is important for IoT systems to have architectures, systems principles, and operations that can accommodate the interesting scale, safety, reliability, and privacy requirements.

Some examples of the application of IoT include:

- Logistics applications, fleet telemetry and supply chain management; the tracking of physical objects such as packages and containers

- Manufacturing and Industrial applications, involving control and operation of industrial equipment and smart production lines

- Asset management and smart shelving. Connected storage and vending devices.

- Building automation or “smart buildings” where monitoring and control systems are applied to all the systems within a building, facilitating smooth operation of the building and the proactive management and maintenance of the equipment and facilities

- Intelligent transportation systems in particular the management of road and rail transport

- Connected vehicles, involving such capabilities as information feeds to drivers about road status or the use of “black boxes” which assess insurance risks/premiums dynamically

- Smart cities, where monitoring and control of city-wide systems are handled automatically for greater efficiency and to serve citizens better

- Smart grid systems, involving instrumenting the electrical grid at all scales for better management and maintenance of equipment, for optimizing the use of power in the grid and dealing with intermittent power sources such as wind

- Consumer applications, typically based on the use of smart phones and wearable

- Medical applications, such as remote monitoring and treatment of patients

- Retail and “intelligent shopping” – making use of information about the consumer to make offers and to direct the consumer to items of interest

- The smart home – autonomous management of domestic premises, including control of heating systems, the operation of consumer appliances and extending to automation of maintenance and ordering of consumables (food, etc.)

## Fundamental to IoT

Fundamental to IoT are electronic devices that interact with the physical world; sensors that gather information about objects and human activities; actuators that can act on objects. Sensors can take many forms. Devices such as thermometers and accelerometers measure real world characteristics and generate numerical information, whereas cameras and microphones create streams of video and audio information containing more complex information about the real world. Beacons and load sensors are also part of the IoT category. Actuators also take different forms – for example relays which can switch on or off equipment, such as a heater, a piece of manufacturing equipment, or alternatively displays which can be used to inform people such as drivers.

There are several aspects that apply to IoT systems that affect their architecture and implementation, as follows:

- Scalability: Scale for IOT system applies in terms of the numbers of sensors and actuators connected to the system, in terms of the networks which connect them together, in terms of the amount of data associated with the system and its speed of movement and also in terms of the amount of processing power required.

- Big Data: Many more advanced IoT systems depend on the analysis of vast quantities of data. There is a need, for example, to extract patterns from historical data that can be used to drive decisions about future actions. The extraction of useful information from complex data such as video is another example of analysis requiring large amounts of processing. The ability to mine existing data for new insights and the need to combine different datasets in novel ways are characteristics likely to be part of an IoT system. IoT systems are thus often classic examples of “Big Data” processing.

- Cloud computing: IoT systems frequently involve the use of cloud computing platforms. Cloud computing platforms offer the potential to use large amounts of resources, both in terms of the storage of data and also in the ability to bring flexible and scalable processing resources to the analysis of data. IoT systems are likely to require the use of a variety of processing software – and the adaptability of cloud services is likely to be required in order to deal with new requirements, firmware or system updates and offer new capabilities over time.

- Real time: IoT systems often function in real time; data flows in continually about events in progress and there can be a need to produce timely responses to that stream of events. This may involve stream processing; acting on the event data as it arrives, comparing it against previous events and also against static data in order to react in the most appropriate way. There is a parallel need to ensure that corrupted data is detected and not used – whether introduced by faulty sensors or malicious action – since the use of corrupted data could cause harm and damage to humans, equipment, and the environment.

- Highly distributed: IoT systems can span whole buildings, span whole cities, and even span the globe. Wide distribution can also apply to data – which can be stored at the edge of the network or stored centrally. Distribution can also apply to processing – some processing takes place centrally (in cloud services), but processing can take place at the edge of the network, either in the IoT gateways or even within (more capable types of) sensors and actuators. Today there are officially more mobile devices than people in the world. Mobile devices and networks are one of the best known IoT devices and networks.

- Heterogeneous systems: IoT systems are often built using a very heterogeneous set of. This applies to the sensors and actuators, but also applies to the types of networks involved and the variety of processing components. It is common for sensors to be low-power devices, and it is often the case that these devices use specialized local networks to communicate. To enable internet scale access to devices of this kind, an IoT gateway is used.

- Security and Privacy: The question of the security and trustworthiness of distributed heterogeneous IoT systems is a hard problem whose solutions must scale and evolve with the systems. Data protection is necessary, including significant privacy concerns regarding data that relate to individuals. Gaining assurance that these systems are safe, secure, resilient and uphold their stakeholder’s expectations about privacy is especially challenging.

- Compliance: Providing confidence about the operation of these IoT systems is necessary both due to the regulations of specific industries, sectors and verticals and also the norms and expectations of the stakeholders of the IoT systems.

- Integration: IoT systems do not exist on their own but need to connect to existing operational technology systems like factory systems, building control systems, and other types of physical management systems as well as existing enterprise systems including enterprise applications and enterprise databases.

Understanding IoT architectures builds on understanding mobile, web application hosting, and big data and analytics capabilities, please refer to Cloud Customer Architecture for Big Data and Analytics, Web Application Hosting and Mobile for a thorough discussion and best practices on each specific topic.

Figure 1 shows the elements that may be needed for any IoT solution across five domains: user layer, proximity network, public networks, provider clouds, and enterprise networks.

Figure 14 Elements of IoT Solutions

## Aspects of the architecture include:

- The user layer is independent of any specific network domain. It may be in or outside any specific domain.

- The proximity network domain has networking capabilities that typically extend the public network domain. The devices (including sensor/actuator, firmware and management agent) and the physical entity are part of the proximity network domain. The devices communicate for both data flow and control flow either via an IoT Gateway and edge services or directly over the public network via edge services.

- The public network and enterprise network domains contain data sources that feed the entire architecture. Data sources include traditional systems of record from the enterprise as well as new sources from Internet of Things (IoT). The public network includes communication with peer clouds.

- The provider cloud captures data from devices, peer cloud services and other data sources (for example Weather services). It can use integration technologies or stream processing to transform, filter and analyses this data in real time and it can store the data into repositories where further analytics can be performed. This processing, which can be augmented with the use of Cognitive and Predictive analytics, is used to generate Actionable Insights. These insights are used by users and enterprise applications and can also be used to trigger actions to be performed by IoT Actuators. All of this needs to be done in a secure and governed environment.

- Results are delivered to users and applications using transformation and connectivity components that provide secure messaging and translations to and from systems of engagement, enterprise data, and enterprise applications.

One of the features of IoT systems is the need for application logic and control logic in a hierarchy of locations, depending on the timescales involved and the datasets that need to be brought to bear on the decisions that need to be made. Some code may execute directly in the devices at the very edge of the network, or alternatively in the IoT Gateways close to the devices. Other code executes centrally in the provider cloud services or in the enterprise network. The term “edge computing” is sometimes applied to the case where code executes in the IoT Gateways or the devices. This is sometimes alternatively called “fog computing” to contrast with centralized “cloud computing”, although fog computing can also contain one or more layers below the cloud that each could potentially provide capabilities for a variety of services like analytics. This design allows flexibility in how connectivity and services are designed for optimization and resiliency.

IoT governance and security subsystems span all elements of the architecture to ensure controls and policies for all data and applications are defined and enabled across the system. Compliance is tracked to ensure controls are delivering the expected results.

Figure 15  Cloud Components for IoT

## Cloud Components

- The cloud components of IoT architecture are positioned within a three-tier architecture pattern comprising edge, platform and enterprise tiers, as described in the Industrial Internet Consortium Reference Architecture.

- The edge-tier includes Proximity Networks and Public Networks where data is collected from devices and transmitted to devices. Data flows through the IoT gateway or optionally directly from/to the device then through edge services into the cloud provider via IoT transformation and connectivity.

- The Platform tier is the provider cloud, which receives, processes and analyzes data flows from the edge tier and provides API Management and Visualization. It provides the capability to initiate control commands from the enterprise network to the public network as well.

- The Enterprise tier is represented by the Enterprise Network comprised of Enterprise Data, Enterprise User Directory, and Enterprise Applications. The data flow to and from the enterprise network takes place via a Transformation and Connectivity component. The data collected from structured and non-structured data sources, including real-time data from stream computing, can be stored in the enterprise data.

## User Layer

It contains IoT users and their end user applications.

- IoT User (people/system) - a person or alternatively an automated system that makes use of one or more end user applications to achieve some goal. The IoT User is one of the main beneficiaries of the IoT solution.

- End User Application - domain specific or device specific application. The IoT user may use end user applications that run on smart phones, tablets, PCs or alternatively on specialized IoT devices including control panels.

## Proximity Network

It contains the physical entities that are at the heart of the IoT system, along with the devices that interact with the physical entities and connect them to the IoT system.

    - Physical Entity

The physical entity is the real-world object that is of interest – it is subject to sensor measurements or to actuator behavior. It is the “thing” in the Internet of Things. This architecture distinguishes between the physical entities and the IT devices that sense them or act on them. For example, the thing can be the ocean and the device observing is it a water temperature thermometer. Another example is a depot shipping parcels: the parcels are the physical entities and there are devices with sensors capable of observing and identifying each parcel – e.g. via RFID tags or via Barcode readers. It is clear that the RFID Tag reader is one thing and the parcels are something completely different - the identity of the parcel is the physical entity here.

    - Device

It contains sensor(s) and/or actuator(s) plus a network connection that enables interaction with the wider IoT system. There are cases where the device is also the physical entity being monitored by the sensors – such as an accelerometer inside a smart phone.

- Sensor/Actuator - senses and acts on physical entities. A sensor is a component that senses or measures certain characteristics of the real world and converts them into a digital representation. An actuator is a component that accepts a digital command to act on a physical entity in some way.

- Agent - provides remote management capabilities for the device, supporting a device management protocol that can be used by the Device Management service or IoT management system.

- Firmware - software that provides control, monitoring and data manipulation of engineered products and systems. The firmware contained in devices such as consumer electronics provides the low-level control program for the devices.

- Network Connection - provides the connection from the device to the IoT system. This is often a local network that connects the device with an IoT gateway – low power and low range in many cases to reduce the power demands on the device. However, there are cases where the network connection is direct to the public network and no IoT gateway is required. In IoT systems, a wide range of alternative communication mechanisms are used which include local area networking using low-power, low-range methods, such as Bluetooth, Bluetooth Low Energy (BTLE), and others. It may also include local area networking using Wi-Fi, to wide area networking using 2G, 3G, and 4GLTE.

## User Interface

It allows users to interact with applications, agents, sensors and actuators (optional – some devices have no user interface and all interaction takes place from remote applications over the network).

## IoT Gateway

It acts as a means for connecting one or more devices to the public network (typically the Internet). It is commonly the case that devices have limited network connectivity – they may not be able to connect directly to the Internet. This can be for a number of reasons, including the limitation of power on the device, which can restrict the device to using a low-power local network. The local network enables the devices to communicate with a local IoT Gateway, which is then able to communicate with the public network. The IoT Gateway often has other capabilities, including the ability to filter and intelligently react to data, the ability to send and receive data or commands to and from the Internet, the ability to run application or service logic locally (processing data and executing control logic without the need to communicate to a central location). It can also provide operational efficiency by allowing multiple devices to share a common connection. The IoT Gateway contains the following components:

- App Logic - provides domain specific or IoT solution specific logic that runs on the IoT Gateway. For IoT systems that have Actuators which act on physical entities, a significant capability of the app logic is the provision of control logic which makes decisions on how the actuators should operate, given input from sensors and data of other kinds, either held locally or held centrally.

- Analytics - provides Analytics capability locally rather than in the provider cloud.

- Agent - allows management of the IoT Gateway itself and can also enable management of the attached devices by providing a connection to the provider cloud layer's Device Management service via the device management protocol.

## Device Data Store

It stores data locally. Devices may generate a large amount of data in real time it may need to be stored locally rather than being transmitted to a central location. Data in the device data store can be used by the application logic and analytics capability in the IoT Gateway.

## Provider Cloud

It provides core IoT applications and associated services including storage of device data; analytics; process management for the IoT system; create visualizations of data. Also hosts components for device management including a device registry. Provider Cloud elements include:

- IoT Transformation and Connectivity

- Application Logic

- Visualization

- Analytics

- Process Management

- Device Data Store

- API Management

- Device Management

- Device Registry

- Device Identity Service

- Transformation and Connectivity

A cloud computing environment provides scalability and elasticity to cope with varying data volume, velocity and related processing requirements. Experimentation and iteration using different cloud service configurations is a good way to evolve the IoT system, without upfront capital investment.

## IoT Transformation and Connectivity

It enables secure connectivity to and from IoT devices. This component must be able to handle and perhaps transform high volumes of messages and quickly route them to the right components in the IoT solution. The Transformation and Connectivity component includes the following capabilities:

- Secure Connectivity - provides the secured connectivity which authenticates and authorizes access to the provider cloud.

- Scalable Messaging - provides messaging from and to IoT devices. Scalability of the messaging component is essential to support high data volume applications and applications with highly variable data rates, like weather.

- Scalable Transformation - provides transformation of device IoT data before it gets to provider cloud layer, to provide a form more suitable for processing and analysis. This may include decoding messages that are encrypted, translating a compressed formatted message, and/or normalizing messages from varying devices.

## Application Logic

The core application components, typically coordinating the handling of IoT device data, the execution of other services and supporting end user applications. An Event based programming model with trigger, action and rules is often a good way to write IoT application logic. Application logic can include workflow. Application logic may also include control logic, which determines how to use actuators to affect physical entities, for those IoT systems that have actuators.

## Visualization

It enables users to explore and interact with data from the data repositories, actionable insight applications, or enterprise applications. Visualization capabilities include End user UI, Admin UI & dashboard as sub components.

- End User UI - allows users to communicate and interact with Enterprise applications, analytics results, etc. This also includes internal or customer facing mobile user interfaces.

- Admin UI - enables administrators to access metrics, operation data, and various logs.

- Dashboard - allows users to view various reports. Admin UI and Dashboard are internal facing user interfaces.

  - Analytics

Analytics is the discovery and communication of meaningful patterns of information found in IoT data, to describe, to predict, and to improve business performance. It covers the following capabilities for IoT:

- Analytics Data repository - supports legacy, new and streaming sources, enterprise applications, enterprise data, cleansed data and reference data, as well as output from streaming analytics. Capabilities include: Exploration & Archive (for storing, exploring and augmenting large data sets using a wide variety of tools); Deep Analytics & Modeling (application of statistical models to yield information from large data sets comprised of both unstructured and weakly-structured elements); Interactive Analysis & Reporting (tools to answer business and operations questions over Internet scale datasets); Data Catalog (results from discovery and IT data creation create a consolidated view of information that is reflected in a catalog). See [2] for more information on Big Data and Analytics Reference Architectures for using cloud computing.

- Cognitive - intelligent system that learns at scale, reasons with purpose, analyses to predict, to prescribe, and to discover from massive datasets of interconnected physical, social, enterprise and other entities, and closes the loop with machine-generated advice, assistance and actions, in a manner that self-learns and adapts, for enabling augmented human intelligence through man/ machine collaborations.

- Actionable Insight - insights that ultimately drive actions that may be used by business applications from data collected, processed and stored in the data repositories. Capabilities include: Decision Management (analytics-based and operational); Discovery & Exploration (exploration across a variety of sources to provide business users with new visibility into business performance); Predictive Analytics (extracts information from existing datasets to determine the current state, identify patterns and predict future trends); Analysis & Reporting (reports of operational and warehouse data to business stakeholders and regulators where big data typically increases the scope and depth of available data); Content Analytics (enables businesses to gain insight and understanding from their structured and unstructured content); Planning & Forecasting (enables faster and more efficient development of plans, budgets and forecasts by creating, comparing and evaluating business scenarios).

  - Streaming Computing

It accepts and processes in real time large volumes of highly dynamic, time-sensitive continuous data streams from a variety of inputs such as sensor-based monitoring devices, messaging systems and financial market feeds. Capabilities include:

RealTime Analytical Processing (applying analytic processing and decision making to in-motion and transient data with minimal latency) and Data Augmentation (filtering and diverting in-motion data to data warehouses for deeper background analysis).

  - Process Management

The activities of planning, developing, deploying and monitoring the performance of a business process. For IoT systems, real-time process management may provide significant benefits.

  - Device Data Store

It stores data from the IoT devices so that the data can be integrated with processes and applications that are part of the IoT System. Devices may generate a large amount of data in real time calling for the Device Data Store to be elastic and scalable.

  - API Management

It publishes catalogues and updates APIs in a wide variety of deployment environments. This enables developers and end users to rapidly assemble solutions through discovery and reuse of existing data, analytics and services.

  - Device Management

It provides an efficient way to manage and connect devices securely and reliably to the cloud platform. Device management contains device provisioning, remote administration, software updating, remote control of devices, monitoring devices. Device management may communicate with management agents on devices using management protocols as well as communicate with management systems for the IoT solutions.

  - Device Registry

It stores information about devices that the IoT system may read, communicate with, control, provision or manage. Devices may need to be registered before they can connect to and or be managed by the IoT system. IoT deployments may have a large number of devices therefore scalability of the registry is important.

  - Device Identity Service

It ensures that devices are securely identified before being granted access to the IoT systems and applications. In the IoT systems, device identification can help address threats that arise from fake servers or fake devices.

  - Transformation and Connectivity

It enables secure connections to enterprise systems and the ability to filter, aggregate, or modify data or its format as it moves between cloud and IoT systems components and enterprise systems (typically systems of record). Within the IoT reference architecture the transformation and connectivity component sit between the cloud provider and enterprise network. However, in a hybrid cloud model these lines might become blurred. The Transformation and Connectivity component includes the following capabilities:

- Enterprise Secure Connectivity - integrates with enterprise data security systems to authenticate and authorize access to enterprise systems.

- Transformation - transforms data going to and from enterprise systems.

- Enterprise Data Connectivity - enables provider cloud components to connect securely to enterprise data. Examples include VPN and gateway tunnels.

  - Security and Privacy

Security and Privacy in IoT deployments must address both information technology (IT) security as well as operations technology (OT) security elements. Furthermore, the level of attention to security and the topic areas to address varies depending upon the application environment, business pattern, and risk assessment. A risk assessment will take into account multiple threats and attacks along with an estimate of the potential costs associated with such attacks.

In addition to security considerations, the connecting of IT systems with physical systems also brings with it the need to consider the impact to safety that the IoT system may have. IoT systems must be designed, deployed, and managed such that they can always bring the system to a safe operating state, even when disconnected from communications with other systems that are part of the deployment. Indeed, disconnecting from communications may be part of the security measures put in place to help secure the IoT deployment.

There are several areas of security to consider:

- Identity and Access Management

- Data Protection

- Security Monitoring, Analysis, and Response

- System, Application, and Solution Lifecycle Management

Each of these areas is briefly discussed below.

    - Identity and Access Management

As with any computing system, there must be strong identification of all participating entities – users, systems, applications, and, in the case of IoT, devices and the IoT gateways through which those devices communicate with the rest of the system. Device identity and management necessarily involves multiple entities, starting with chip and device manufacturers, including IoT platform providers, and also including enterprise users and operators of the devices. In IoT solutions it is often the case that multiple of these entities will continue to communicate and address the IoT devices throughout their operational lifetime.

    - Data Protection

Data in the device, in flight throughout the public network, provider cloud, and enterprise network, as well as at rest in a variety of locations and formats must be protected from inappropriate access and use. Multiple methods can be utilized, and indeed, in many cases, multiple methods are applied simultaneously to provide different levels of protection of data against different types of threats or isolation from different entities supporting the system. Communications link protection may be used in addition to individual data field level encryption and/or signing done at/in the device in order to provide both end-to-end and point-to-point communications protection. Data at rest in different formats may be encrypted at the field, database, and even whole disk/media level to protect against leakage and improper usage. Increased data collection also results in a need to consider potential privacy implications, requiring additional attention to data segregation, redaction, and special handling requirements.

    - Security Monitoring, Analysis, and Response

Every system must have monitoring of the environment built in so that active attacks as well as anomalous behavior is detected and acted upon. Because of the scale of IoT systems, both in the number of devices as well as the amount of information being processed, there is a requirement for automated response to known attacks as well as automatic detection of suspicious behavior. Response to attacks and suspicious behavior may include temporary isolation, quarantine, or removal of parts of the IoT system as well as having formal incident response processes for addressing vulnerabilities that are discovered after the systems have been put into service. Like IT security, there is a need for disclosure of vulnerabilities such that all affected parties can implement appropriate mitigations, changes, and updates in a timely manner. Note that attacks could come in a wide variety of different forms. As just one example, an attack might come in the form of injection of fake, erroneous, or erratic sensor data into the IoT system in an attempt to steer automated decision-making parts of the system to act in a desired (by the attacker) manner. Such attacks must also be expected, planned for, and responded to.

    - System, Application, and Solution Lifecycle Management

Lifecycle management of the IoT system is complex, multi-faceted, and has relationships with identity management, device management, the supply chain, application and software development, through to system operations and change management of deployed and in-service systems. Attention to security in all of these areas is required in order to prevent a variety of attacks ranging from malicious code insertion to inappropriate firmware/software deployment, to effective cryptographic key management. Code, key material, and even physical components must be verified as they flow from procurement and creation through to their installation into the devices, IoT gateways, and systems that make up the IoT system. The IoT system should also provide the capability to update individual components in a secure way, both to address vulnerabilities and also to address functional enhancements over the lifetime of the system. Similar lifecycle considerations apply to privacy and data protection – from “privacy by design” approaches during the creation and assembly of the IoT system through to data deletion when components are decommissioned.

  - Deployment Considerations

Cloud environments offer tremendous flexibility with less concern for how components are physically connected. The need for advanced planning is reduced but still important. This section offers suggestions for better provisioning of data and computing resources.

Initial Criteria

- Scalability & Elasticity

- Data Bandwidth

- Data Sovereignty

- Resilience

- CPU and Computation

- Data Volume

- Security

Figure 16  Complete Picture of IoT Cloud

No single cloud environment optimizes all these criteria. A little advanced planning goes a long way towards ensuring user satisfaction – and it helps keep costs in line with expectations.

    - Scalability & Elasticity

In IoT architecture the number of sensors can be very large and the associated number of transactions can be even larger. This is further multiplied in cases such as connected cars with other data such as traffic and the weather. IoT transformation and connectivity needs to provide scalable messaging and scalable transformation of data in cloud for these data flows. Elasticity is the ability for a cloud solution to provision and de-provision computing resources on demand as workloads change. Public clouds have a distinct advantage since they generally have larger pools of resources available. You also benefit by only paying for what you use. Private clouds and dedicated hardware can make up some of the difference with higher bandwidth data paths. Setting up auto-scale for the queue is not necessarily a one-time event; adjust as usage is better understood to avoid over or under subscribing.

    - Data Bandwidth

Public and private clouds need to be optimized for big data. Large cloud data sets requiring fast access benefit from processing components with fast and efficient data access. In many cases, this means moving the processing to the data, or vice versa. Cloud systems can effectively hide the physical location of data and processing. Tuning activities can be carried out continuously with minimal impact on deployed applications.

    - Data Sovereignty

The physical location in which data is stored may be regulated, with the regulations varying from country to country. This is particularly the case for personally identifiable information (PII) and for sensitive data such as health data and financial records. The European Union has particularly stringent regulations that apply to the PII of European citizens. As a result, any IoT cloud system must take into account data sovereignty rules and store and process data only in those locations permitted by the regulations – this requires that the provider cloud used provides the cloud service customer with control over storage and processing locations.

    - Resilience

In IoT systems resilience and fault tolerance is very important. IoT systems should not depend on one single component at any point and should tolerate the failure of a single component, such as a single IoT device. Components in the provider cloud can be made resilient through the use of multiple instances of programs and cloud services allied with data replication and redundancy on multiple storage systems. The networks should also be resilient, for example with multiple paths and multiple providers in the public network. There is no silver bullet to make the entire network available all the time but it should be a highly available and resilient. It is important to ensure that the connectivity capabilities can support resilience.

    - CPU and Computation

The availability of inexpensive commodity processors means that public, private and hybrid cloud server farms are typically highly scalable. Modern development environments using Hadoop, Spark and Jupiter (iPython) take advantage of these massively parallel systems. Streams and high-speed analytics are an emerging area where cloud applications leverage more powerful processor pools to enable real-time, in-motion data solutions. Dedicated hardware allows for faster development and testing prior to migration towards hybrid and public environments.

    - Data Volume

In IoT systems the data volume can exceed a threshold at which the traditional analytic toolsets and approaches may no longer scale in meeting performance requirements. So careful planning to store data in public cloud or private cloud or traditional data center is very important. Streaming of data in cases of weather or map use for GPS may result in a huge data set for analysis. Also, all data loses relevance over time. Data retention requires a little experimentation, unless specifically governed by regulatory or other policies. Public clouds offer the flexibility to store varying amounts of data with no advance provisioning. In-house cloud storage solutions can offer long term storage cost advantages when volume is predicted in advance.

    - Security

As more data about people, financial transactions and operational decisions is collected, refined and stored, the challenges related to information governance and security increase. The data privacy and identity management of devices and individual is very important from the cloud computing point of view. The cloud generally allows for faster deployment of new compliance and monitoring tools that encourage agile policy and compliance frameworks. Cloud data hubs can be a good option by acting as focal points for data assembly and distribution. Tools that monitor activity and data access can actually make cloud systems more secure than standalone systems. Hybrid systems offer unique application governance features: Software can be centrally maintained in a distributed environment with data stored in-house to meet jurisdictional policies.

# CHAPTER 4

## NODEMCU and Its Configuration with Arduino IDE

Development boards, such as Arduino and Raspberry Pi, are common choices when prototyping new IoT devices. Those development boards are essentially mini-computers that can connect to and be programmed by a standard PC or Mac. After it has been programmed, the development boards can then connect to and control sensors in the field. Because the "I" in IoT stands for internet, the development boards need a way to connect to the internet. In the field, the best way to connect to the internet is by using wireless networks. However, Arduino and Raspberry Pi do not have built-in support for wireless networks. Developers will have to add a Wi-Fi or cellular module to the board and write code to access the wireless module.

In this article, I will introduce an open source IoT development board called NodeMCU. One of its most unique features is that it has built-in support for Wi-Fi connectivity, and hence makes IoT application development much easier.

## NodeMCU

The NodeMCU (Node Microcontroller Unit) is an open source software and hardware development environment that is built around a very inexpensive System-on-a-Chip (SoC) called the ESP8266. The ESP8266, designed and manufactured by Espressif Systems, contains all crucial elements of the modern computer: CPU, RAM, networking (Wi-Fi), and even a modern operating system and SDK. When purchased at bulk, the ESP8266 chip costs only $2 USD a piece. That makes it an excellent choice for IoT projects of all kinds.

However, as a chip, the ESP8266 is also hard to access and use. You have to solder wires, with the appropriate analog voltage, to its PINs for the simplest tasks such as powering it on or sending a keystroke to the "computer" on the chip. And, you have to program it in low-level machine instructions that can be interpreted by the chip hardware. While this level of integration is not a problem when the ESP8266 is used as an embedded controller chip in mass-produced electronics, it is a huge burden for hobbyists, hackers, or students who want to experiment with it in their own IoT projects.

Borrowing a page from the successful playbooks of Arduino or a Raspberry Pi, the NodeMCU project aims to simplify ESP8266 development. It has two key components.

- An open source ESP8266 firmware that is built on top of the chip manufacturer's proprietary SDK. The firmware provides a simple programming environment based on eLua (embedded Lua), which is a very simple and fast scripting language with an established developer community. For new comers, the Lua scripting language is easy to learn.

- A DEVKIT board that incorporates the ESP8266 chip on a standard circuit board. The board has a built-in USB port that is already wired up with the chip, a hardware reset button, Wi-Fi antenna, LED lights, and standard-sized GPIO (General Purpose Input Output) pins that can plug into a bread board. Figure 1 shows the DEVKIT board, and Figure 2 shows the schema of its pins.

Figure 17  NodeMCU PINs Details

The NodeMCU DEVKIT board that comes preloaded with the firmware can be purchased for $8 USD a piece, which makes it a very economical device for prototyping and even for production use.

But, what about Arduino, you ask? The Arduino project creates an open source hardware design and software SDK for a versatile IoT controller. Similar to NodeMCU, the Arduino hardware is a microcontroller board with a ready USB connector, LED lights, and standard data pins. It also defines standard interfaces to interact with sensors or other boards. But unlike NodeMCU, the Arduino board can have different types of CPU chips (typically an ARM or Intel x86 chip) with memory chips, and a variety of programming environments. In fact, there is an Arduino reference design for the ESP8266 chip as well. However, the flexibility of Arduino also means significant variations across different vendors. For example, most Arduino boards do not have Wi-Fi capabilities and some even have a serial data port instead of a USB port. I feel that NodeMCU provides a more consistent and accessible experience for IoT developers.

## NodeMCU Specification

- Voltage:3.3V.

- Wi-Fi Direct (P2P), soft-AP.

- Current consumption: 10uA~170mA.

- Flash memory attachable: 16MB max (512K normal).

- Integrated TCP/IP protocol stack.

- Processor: Tensilica L106 32-bit.

- Processor speed: 80~160MHz.

- RAM: 32K + 80K.

- GPIOs: 17 (multiplexed with other functions).

- Analog to Digital: 1 input with 1024 step resolution.

- +19.5dBm output power in 802.11b mode

- 802.11 support: b/g/n.

- Maximum concurrent TCP connections: 5.

The most basic way to use the ESP8266 module is to use serial commands, as the chip is basically a Wi-Fi/Serial transceiver. However, this is not convenient. What we recommend is using the very cool Arduino ESP8266 project, which is a modified version of the Arduino IDE that you need to install on your computer. This makes it very convenient to use the ESP8266 chip as we will be using the well-known Arduino IDE. Following the below step to install ESP8266 library to work in Arduino IDE environment.

## Install the Arduino IDE 1.6.4 or greater

Download Arduino IDE from Arduino.cc (1.6.4 or greater) - don't use 1.6.2 or lower version! You can use your existing IDE if you have already installed it.

You can also try downloading the ready-to-go package from the ESP8266-Arduino project, if the proxy is giving you problems.

## Install the ESP8266 Board Package

Enter http://arduino.esp8266.com/stable/package_esp8266com_index.json into Additional Board Manager URLs field in the Arduino v1.6.4+ preferences.

## NodeMCU Configuration with Arduino IDE

There are following steps to be followed to configure NodeMCU with Arduino IDE

  - Download the Latest Arduino IDE from the Arduino website

  - Install the Arduino IDE

  - Open the Arduino IDE

  - Click on File Tab

    - Go to Preferences

    - Find Additional Board Managers URLs

    - Copy the following link in it-

      - http://arduino.esp8266.com/stable/package_esp8266com_index.json

    - Click OK to Close preference tab.

Figure 18  Adding NodeMCU URL

Next, use the Board manager to install the ESP8266 package.

Figure 19  NodeMCU Packages Installation

Figure 20  ESP8266 Packages Installation

  - Go to Tools Tab

    - Select Board

      - Board Manager

    - Some packages will be downloaded from internet automatically

    - Type ESP8266

      - Esp8266 by esp8266 community package will appear

      - Install this package

    - Go to Tools and then Board. Here search for NodeMCU 1.0.

Set Up NodeMCU with Arduino:

Selection of Frequency for NodeMCU:

Go to Tools

- CPU Frequency

- Select 80 MHz

Figure 21  Selection of NodeMCU

Figure 22  Selection of Frequency

Selection of Baud Rate:

Select ‘115200’ baud upload speed is a good place to start - later on you can try higher speeds but 115200 is a good safe place to start.

Go to Tools

- Select Upload Speed

- Select 115200

There are many baud rates are available, that can be selcted and used as per the requirement.

Figure 23 Selection of Baud Rate

Setting Up of COM Port:

Go to your Windows ‘Device Manager’ to find out which Com Port ‘USB-Serial CH340’ is assigned to. Select the matching COM/serial port for your CH340 USB-Serial interface.

Go to My Computer/ My PC Icon

- Right Click

- Properties

- Device Manager

- Ports (COM & LPT)

- Expand it

- USB Serial (COM#)

Figure 24  COM Poet Seeting in Device Manager

Find same COM Port in Arduino Ide

- Open Ardunio Ide

- Tools

- Port

- Selct COM #

Figure 25  COM Port Selection in Arduino Ide

Testing of NodeMCU SetUp

Testing the setup, the simple blink test program will be flashed into NodeMCU. Enter the below given program into Arduino sketch window and save it into desired folder.

/*

ESP8266 Blink

Blink the blue LED on the ESP8266 module

*/

#define LED 2 //Define blinking LED pin

void setup () {

pin Mode(LED, OUTPUT); // Initialize the LED pin as an output

}

// the loop function runs over and over again forever

void loop() {

digitalWrite(LED, LOW); // Turn the LED on (Note that LOW is the voltage level)

delay(1000); // Wait for a second

digitalWrite(LED, HIGH); // Turn the LED off by making the voltage HIGH

delay(1000); // Wait for two seconds

}

Click on the Verify Button  for Compilling the Sketch

Figure 26  Verification of Sketch

After successful compilation click on the Upload Button and wait for 100% uploading of program into the NodeMCU.

Figure 27  Uploading of Sketch into NodeMCU

After Sucessful uploading of program into NODEMCU, the Built-in LED will of Development Kit will start blinking with 1 second delay.

# CHAPTER 5

## Introduction and Interfacing of Blynk App with Device

## Introduction to Blynk

Blynk was designed for the Internet of Things. It can control hardware remotely, it can display sensor data, it can store data, visualize it and do many other cool things.

There are three major components in the platform:

- Blynk App - allows to you create amazing interfaces for your projects using various widgets we provide.

- Blynk Server - responsible for all the communications between the smartphone and hardware. You can use our Blynk Cloud or run your private Blynk server locally. It’s open-source, could easily handle thousands of devices and can even be launched on a Raspberry Pi.

- Blynk Libraries - for all the popular hardware platforms - enable communication with the server and process all the incoming and outcoming commands.

Now imagine: every time you press a Button in the Blynk app, the message travels to the Blynk Cloud, where it magically finds its way to your hardware. It works the same in the opposite direction and everything happens in a Blynk of an eye.

Figure 28 Blynk Cloud Architecture

## Features

- Similar API & UI for all supported hardware & devices

- Connection to the cloud using:

- Wi-Fi

- Bluetooth and BLE

- Ethernet

- USB (Serial)

- GSM

- Set of easy-to-use Widgets

- Direct pin manipulation with no code writing

- Easy to integrate and add new functionality using virtual pins

- History data monitoring via History Graph widget

- Device-to-Device communication using Bridge Widget

- Sending emails, tweets, push notifications, etc.

## Blynk Configuring

Just a couple of things are required for configuring Blynk for our application:

The required items are

- Hardware with Wi-Fi

- Smartphone with Blynk app installed

#### 1. Hardware.

There are following hardware may be used with Blynk An Arduino, Raspberry Pi,

NodeMCU or a similar development kit.

Blynk works over the Internet. This means that the hardware you choose should be able to connect to the internet. Some of the boards, like Arduino Uno will need an Ethernet or Wi-Fi Shield to communicate, others are already Internet-enabled: like the ESP8266, Raspberry Pi with Wi-Fi dongle, Particle Photon or Spark Fun Blynk Board. But even if you don’t have a shield, you can connect it over USB to your laptop or desktop (it’s a bit more complicated for newbies, but we got you covered).

#### 2. A Smartphone.

The Blynk App is a well-designed interface builder. It works on both kind of

operating systems iOS and Android.

## Getting Started with The Blynk App

User must be following the below given procedure for setting up the Blynk cloud

## Create a Blynk Account

- After you download the Blynk App, you’ll need to create a New Blynk account. This account is separate from the accounts used for the Blynk Forums, in case you already have one.

- We recommend using a real email address because it will simplify things later.

Figure 29  New Account Creation

- An account is needed to save your projects and have access to them from multiple devices from anywhere. It’s also a security measure.

- You can always set up your own Private Blynk Server and have full control.

## Create a New Project

- After you’ve successfully logged into your account, start by creating a new project and give it a name to it as shown in below given figures

- Now Select the development board model from drop down list of Hardware Model

- At last click on Cerate tab

Figure 30  New Project Creation

Figure 31  New Project Name and Device Selection

## Authentication Token

Auth Token is a unique identifier which is needed to connect your hardware to your smartphone. Every new project you create will have its own Auth Token. You’ll get Auth Token automatically on your email after project creation. You can also copy it manually.

- Click on devices section

- Click on Device

- Now Auth Token will appear on canvas

Figure 32  New Project Setting

Figure 33   Addition of More Device in same project

Figure 34  Authentication Token

It’s very convenient to send it over e-mail. Press the

e-mail button and the token will be sent to the e-mail address you used for registration. You can also tap on the Token line and it will be copied to the clipboard.

Now press the “Create” button.

Figure 35  Project Creation

## Add a Widget

Your project canvas is empty, let’s add a button to control our LED.

Tap anywhere on the canvas to open the widget box. All the available widgets are located here. Now pick a button.

Widget Box

- Button Symbol will appear on canvas

- Drag-n-Drop - Tap and hold the Widget to drag it to the new position

Figure 36  Blynk Widgets

- Widget Settings - Each Widget has its own settings. Tap on the widget to get to them.

Figure 37  Blynk Widgets Button Selection

Figure 38  Blynk Widgets Button Settings

- The most important parameter to set is PIN. The list of pins reflects physical pins defined by your hardware. If your LED is connected to Digital Pin 8 - then select D8 (D - stands for Digital).

## Run Project

When you are done with the Settings - press the PLAY button. This will switch you from EDIT mode to PLAY mode where you can interact with the hardware. While in PLAY mode, you won’t be able to drag or set up new widgets, press STOP and get back to EDIT mode.

You will get a message saying “NODEMCU is offline”. We’ll deal with that in the next section.

Figure 39  Blynk Widgets Button online

## Virtual Pin Operations

Blynk can control Digital and Analog I/O Pins on your hardware directly. You don’t even need to write code for it. It’s great for blinking LEDs, but often it’s just not enough.

We designed Virtual Pins to send any data from your microcontroller to the Blynk App and back. Anything you connect to your hardware will be able to talk to Blynk. With Virtual Pins you can send something from the App, process it on microcontroller and then send it back to the smartphone. You can trigger functions, read I2C devices, convert values, control servo and DC motors etc.

Virtual Pins can be used to interface with external libraries (Servo, LCD and others) and implement custom functionality. Hardware may send data to the Widgets over the Virtual Pin like this:

Blynk.virtualWrite(pin, "abc");

Blynk.virtualWrite(pin, 123);

Blynk.virtualWrite(pin, 12.34);

Blynk.virtualWrite(pin, "hello", 123, 12.34);

Make sure you differentiate Virtual Pins from physical pins on your hardware. Virtual Pins have no physical representation.

Virtual Pins can be used to interface with libraries (Servo, LCD and others) and implement custom functionality. The device may send data to the App using Blynk.virtualWrite(pin, value) and receive data from the App using BLYNK_WRITE(vPIN).

## Sending Data from APP to Hardware

You can send any data from Widgets in the app to your hardware. All Controller Widgets can send data to Virtual Pins on your hardware. For example, code below shows how to get values from the Button Widget in the App

BLYNK_WRITE(V1) //Button Widget is writing to pin V1

{

int pinData = param.asInt();

}

When you press a Button, Blynk App sends 1

On the second click - it sends 0

This is how Button Widget is set up:

Figure 40  Blynk Widgets Button Selection

## Pushing Data from Hardware

If you need to PUSH sensor or other data from your hardware to Widget, you can write any logic you want. Just set the frequency to PUSH mode. Any command that hardware sends to Blynk Cloud is automatically stored on server and you get this info either with History Graph widget or with HTTP API.

Figure 41  Blynk Widget Pushing Data

## Controlling of Multiple Devices

Blynk app has support of multiple devices. That means you can assign any widget to specific device with own Auth token. For example - you may have button on V1 that controls wi-fi bulb A and another button on V1 that controls wi-fi bulb B. In order to do this, you need more than 1 device within your project. To achieve this please go to project settings and click on “Devices” section:

Figure 42  Blynk Widgets Button Selection

## Widgets

Widgets are interface modules. Each of them performs a specific input/ output function when communicating with the hardware.

There are 4 types of Widgets:

    - Controllers - they send commands to hardware. Use them to control your stuff;

    - Displays - used for various visualizations of data that comes from hardware to the smartphone;

    - Notifications - are various widgets to send messages and notifications;

    - Interface - are various widgets to make your UI look better;

    - Others - widgets that don’t belong to any category;

Each Widget has its own settings. Some of the Widgets (e.g. Bridge Widget) are

used to enable some functionality and they don’t have any settings.

## Common Widgets Setting

  - Pin Selector- This is one of the main parameters you need to set. It defines which pin to control or to read from.

    - Digital Pins - represent physical Digital IO pins on your hardware. PWM-enabled pins are marked with the ~ symbol

    - Analog Pins - represent physical Analog IO pins on your hardware.

    - Virtual Pins - have no physical representation. They are used for any data transfer between Blynk App and your hardware.

  - Data Mapping- in case you want to map incoming values to specific range you may use mapping button. Let’s say your sensor sends values from 0 to 1023. But you want to display values in a range 0 to 100 in the application. With data mapping enabled, incoming value 1023 will be mapped to 100.

  - Split/Merge - Some of the Widgets can send more than one value. And with this switch you can control how to send them.

    - SPLIT: Each of the parameters is sent directly to the Pin on your hardware (e.g. D7). You don’t need to write any code

    - MERGE: When MERGE mode is selected, you are sending just 1 message, consisting of array of values. But you’ll need to parse it on the hardware. This mode can be used with Virtual Pins only.

## Controllers

    - Buttons

Works in push or switch modes. Allows to send 0/1 (LOW/HIGH) values. Button sends 1 (HIGH) on press and sends 0 (LOW) on release.

    - Slider

Similar to potentiometer. Allows to send values between MIN and MAX.

    - Timer

Timer triggers actions at a specific time. Even if smartphone is offline. Start time sends 1 (HIGH). Stop time sends 0 (LOW). Recent Android version also has improved Timer within Eventer widget. With Eventer Time Event you can assign multiple timers on same pin, send any string/value, select days and time zone. It is recommended to use Eventer over Timer widget. However, Timer widget is still suitable for simple timer events.

    - Joystick

Control servo movements in 4 directions

    - Step Control - Step control is like 2 buttons assigned to 1 pin. One button increments your value by desired step and another one decrements it. It is very useful for use cases where you need to change your values very precisely and you can’t achieve this precision with slider widget. Send Step option allows you to send step to hardware instead of actual value of step widget. Loop value option allows you to reset step widget to start value when maximum value is reached.

  - Notifications

    - Twitter

Twitter widget connects your Twitter account to Blynk and allows you to send Tweets from your hardware.

    - Email

Email widget allows you to send email from your hardware to any address.

    - Limitations

    - Maximum allowed email + subject + message length is 120 symbols. However, you can increase this limit if necessary by adding #define BLYNK_MAX_SENDBYTES XXX to you sketch. Where XXX is desired max length of your email. For example, for ESP you can set this to 1200 max length #define BLYNK_MAX_SENDBYTES 1200. The #define BLYNK_MAX_SENDBYTES 1200 must be included before any of the Blynk includes.

    - Only 1 email per 15 seconds is allowed

    - In case you are using Gmail on the Local Server you are limited with 500 mails per day (by google). Other providers may have similar limitations, so please be careful.

    - User is limited with 100 messages per day in the Blynk Cloud;

    - Push Notification

Push Notification widget allows you to send push notification from your hardware to your device. Currently it also contains 2 additional options.

      - Notify when hardware offline

you will get push notification in case your hardware went offline.

      - Offline Ignore Period - defines how long hardware could be offline (after it went offline) before sending notification. In case period is exceeded - “hardware offline” notification will be send. You will get no notification in case hardware was reconnected within specified period.

      - Priority high priority gives more chances that your message will be delivered without any delays.

### CONCLUSION

The automatic sorting machine using conveyor belt is basically useful for sorting the products in the industry specifically large scale industries where mass production is carried out. The machine also reduces the efforts of the workers by reducing the time spent for material handling. The application area of this machine is very wide in industries where automation is built

Future Scope

By producing such system, the scope is in the field of

- Manufacturing

- Identification of suitable solution

- Evaluation

- Design the proposal

- Build the prototype [6].

Also we can use such system with some modification for various types of inspection such as

Inspection Parameter

- Diameter, hole diameter

- Height

- Thickness

- Surface defect

- Crack, burr

- Roundness

- Minor and major diameter

•.

### REFERENCES

[1] Zulhashikin Bin Talib, “Design and Modelling of Automated Sorting System in Manufacturing Industry Using Simlation Software,” Thesis of Universiti Teknikal Malaysia, Meleka, May 2007.

[2] Pankaj Agrawal, Ratnesh kumar Tiwari, Samardeep Banyal, Santosh Kanigicherla, Shivang Chaudhary, “Color/Metal Sensing Sorting System”, International Journal Of Engineering Research & Management Technology (IJERMT), Volume 1, Issue 2, March 2014.

[3] Apple, Material handling equipment, Ronald Press Co., 1972.

[4] Alexandrov, Material handling equipment, central books limited, 1981.

[5] M.A.Alspaugh, Conveyor belt technologies, 2008.

[6] Zulhashikin Bin Talib, “Design and Modelling of Automated Sorting System in Manufacturing Industry Using Simlation Software,” Thesis of Universiti Teknikal Malaysia, Meleka, May 2007.

[7] Prof. D.B.Rane, Gunjal Sagar, Nikam Devendra V., Shaikh Jameer U., “Automation of Object Sorting Using an Roboarm and MATLAB Based Image Processing”, International Journal Of Emerging Technology and Advanced Engineering (IJETAE), Volume 5, Issue 2, February 2015.

## Appendix-A

## Source Code

#define BLYNK_PRINT Serial

#include <ESP8266WiFi.h>

#include <BlynkSimpleEsp8266.h>

#include <Servo.h>  // Include the Servo library

Servo servo_motor;

// You should get Auth Token in the Blynk App.

// Go to the Project Settings (nut icon).

char auth[] = "b36c653424b049088a5ae7e332a95b4";

// Your WiFi credentials.

// Set password to "" for open networks.

char ssid[] = "Digisol";

char pass[] = "";

int ir1=12; //Digital pin D6

int ir2=16; //Digital pin D7

int servoPin = 2 ;//Digital pin D4

int small=0;

int big=0;

int blynk_pin = 0;

void setup() {

// put your setup code here, to run once:

Serial.begin(115200);

Blynk.begin(auth, ssid, pass);

pinMode(ir1,INPUT);

pinMode(ir2,INPUT);

servo_motor.attach(servoPin); // We need to attach the servo to the used pin number

servo_motor.write(90);  //set servo at 90 degrees default

Blynk.virtualWrite(V2,big);

Blynk.virtualWrite(V3,small);

}

BLYNK_WRITE(V1){

if (param.asInt()){

blynk_pin=1;

}

else{

blynk_pin=0;

}

}

void loop() {

// put your main code here, to run repeatedly:

Blynk.run();

if(blynk_pin) {

int ir1_status=digitalRead(ir1);

int ir2_status=digitalRead(ir2);

/* if(ir1_status == 1 && ir2_status == 1){

big = big + 1;

Blynk.virtualWrite(V2,big);

servo_motor.write(90);

//delay(1000);

}*/

if(ir1_status == 1 && ir2_status == 0){

small = small + 1;

Blynk.virtualWrite(V3,small);

servo_motor.write(160);

//delay(1000);

}

else{

servo_motor.write(90);

//delay(1000);

big = big + 1;

Blynk.virtualWrite(V2,big);

}

delay(1000);

}

}